## 🎯 Aim

To write an Arduino program to interface a PIR (Passive Infrared) Sensor with Arduino and observe its behavior under different motion conditions.

## 🧩 Outcome

- Successfully interface and control a PIR sensor using Arduino.

- Detect human/object motion using the infrared heat signature.

- Display output messages in the Serial Monitor to indicate motion detection.

## ⚙️ Hardware Requirements

| Component | Function |
| --- | --- |
| Arduino Uno | Controls the system and reads sensor output. |
| PIR Sensor (HC-SR501 or equivalent) | Detects motion using infrared radiation. |
| Jumper wires | For electrical connections. |
| LED (optional) | Can be added to indicate motion visually. |

## 💻 Software Requirement

- Arduino IDE

## 📖 Theory

### 🔸 What is a PIR Sensor?

- PIR (Passive Infrared Sensor) detects motion by measuring infrared radiation (heat) emitted by humans or animals.

- It has two pyroelectric sensors inside it.

  - When a person moves across its field of view, one sensor detects more IR than the other, creating a voltage difference — indicating motion.

- It does not emit infrared; it only detects it (hence passive).

### ⚙️ PIR Sensor Pin Description

| Pin | Name | Description |
| --- | --- | --- |
| VCC | Power supply (typically 5V) |  |
| OUT | Digital output — HIGH (motion) / LOW (no motion) |  |
| GND | Ground |  |

### ⚙️ Arduino Pin Connections

| PIR Sensor Pin | Arduino Pin | Description |
| --- | --- | --- |
| VCC | 5V | Power supply |
| GND | GND | Ground |
| OUT | D4 | Signal output pin |

## 🔌 Circuit Description

- Connect PIR VCC → Arduino 5V.

- Connect PIR GND → Arduino GND.

- Connect PIR OUT → Arduino Digital Pin 4.

- (Optional) Connect LED to Pin 13 through a 220Ω resistor for motion indication.

When motion is detected, the PIR output pin goes HIGH; when no motion, it stays LOW.

## 💻 Code (from your file)

const int PIR_SENSOR_OUTPUT_PIN = 4;  /* PIR sensor O/P pin */

int warm_up;

void setup() {

pinMode(PIR_SENSOR_OUTPUT_PIN, INPUT);

Serial.begin(9600);   /* Define baud rate for serial communication */

delay(2000);          /* Power On Warm Up Delay */

}

void loop() {

int sensor_output;

sensor_output = digitalRead(PIR_SENSOR_OUTPUT_PIN);

if (sensor_output == LOW)

{

if (warm_up == 1)

{

Serial.print("Warming Up\n\n");

warm_up = 0;

delay(2000);

}

Serial.print("No object in sight\n\n");

delay(1000);

}

else

{

Serial.print("Object detected\n\n");

warm_up = 1;

delay(1000);

}

}

## 🧠 Detailed Explanation (Line-by-Line)

### 1️⃣ Variable Declaration

const int PIR_SENSOR_OUTPUT_PIN = 4;

int warm_up;

- PIR_SENSOR_OUTPUT_PIN → Defines Arduino digital pin 4 for PIR sensor output.

- warm_up → Used to check if the sensor just started detecting movement.

### 2️⃣ setup() Function

void setup() {

pinMode(PIR_SENSOR_OUTPUT_PIN, INPUT);

Serial.begin(9600);

delay(2000);

}

- pinMode(PIR_SENSOR_OUTPUT_PIN, INPUT);
→ Sets the PIR output pin as input so Arduino can read motion data.

- Serial.begin(9600);
→ Initializes serial communication at a speed of 9600 bits/second, allowing the Arduino to print messages to the Serial Monitor.

- delay(2000);
→ Adds a 2-second warm-up delay for the PIR sensor after power ON.
🔹 Why delay?
The PIR sensor needs time (2–5 seconds) to stabilize its infrared readings after power is applied, avoiding false detections.

### 3️⃣ loop() Function

int sensor_output;

sensor_output = digitalRead(PIR_SENSOR_OUTPUT_PIN);

- Reads the digital value from the PIR sensor.

  - HIGH (1) → Motion detected

  - LOW (0) → No motion

### 4️⃣ If No Motion is Detected

if (sensor_output == LOW)

{

if (warm_up == 1)

{

Serial.print("Warming Up\n\n");

warm_up = 0;

delay(2000);

}

Serial.print("No object in sight\n\n");

delay(1000);

}

- If the output is LOW, it means no motion is detected.

- If the sensor was previously detecting motion (warm_up == 1), it prints "Warming Up", then resets the variable to 0.

- It then prints "No object in sight".

- delay(1000) gives a 1-second gap before the next reading.
🔹 Why delay here?
To avoid flooding the Serial Monitor with too many messages and to provide stable output readings once every second.

### 5️⃣ If Motion is Detected

else

{

Serial.print("Object detected\n\n");

warm_up = 1;

delay(1000);

}

- If PIR output is HIGH, motion is detected.

- It prints "Object detected" to the Serial Monitor.

- Sets warm_up = 1 to mark that detection occurred.

- Adds another 1-second delay to prevent multiple detections from one movement.
🔹 Why delay here?
PIR sensors stay HIGH for a short period even after motion stops — this delay prevents false multiple triggers.

## 🖥️ Expected Serial Monitor Output

Example when running:

Warming Up

No object in sight

Object detected

No object in sight

Object detected

## ⚙️ Real-World Behavior

- When you move your hand or walk in front of the sensor → “Object detected” prints.

- When you stand still or move away → “No object in sight” prints.

- On power ON → “Warming Up” appears once during stabilization.

## 📈 Applications

- Motion-activated lighting systems.

- Security alarms and intruder detection.

- Energy-saving automation (turning off unused lights).

- Smart home automation (IIoT systems).

## ✅ Conclusion

The PIR motion sensor was successfully interfaced with Arduino.
The system detects motion accurately and displays detection results in the Serial Monitor.
Delays were added intentionally for sensor stabilization and output readability.

## 🎤 Possible Oral (Viva) Questions

| Question | Expected Answer |
| --- | --- |
| What does PIR stand for? | Passive Infrared Sensor. |
| What type of output does a PIR sensor provide? | Digital output (HIGH or LOW). |
| Why is a delay used after power ON? | To allow the PIR sensor to stabilize. |
| What is the typical voltage required by a PIR sensor? | 5V DC. |
| What will happen if you remove the delay(1000)? | The Serial Monitor will print continuously and be difficult to read; also, the sensor might detect motion repeatedly. |
| How many pins does a PIR sensor have? | Three: VCC, GND, and OUT. |
| What is the use of the “warm_up” variable? | It tracks whether the sensor just detected motion to print "Warming Up" once. |
| What library is required to use a PIR sensor? | None — it works with standard digital I/O functions. |