## 🎯 Aim

To implement IIoT-based robotics and automation by interfacing an Ultrasonic Sensor, IR Sensor, and Buzzer with Arduino to detect obstacles and measure distance.

## 🧩 Outcome

- Establish connectivity and control of multiple sensors (IR + Ultrasonic) with Arduino.

- Detect object distance and presence in real-time.

- Trigger a buzzer alarm when obstacles are too close—an example of industrial IoT monitoring.

## ⚙️ Hardware Components and Functions

| Component | Description |
| --- | --- |
| Arduino UNO | The microcontroller that reads sensor data and controls output devices. |
| Ultrasonic Sensor (HC-SR04) | Measures distance using ultrasonic sound waves. |
| IR Sensor Module | Detects nearby obstacles using infrared reflection. |
| Buzzer | Provides audible alerts when an obstacle is detected within the danger zone. |
| Jumper Wires & Breadboard | For electrical connections. |
| Power Source (USB 5 V) | Supplies power to Arduino and sensors. |

## 🔌 Pin Connections

| Arduino Pin | Connected To | Function |
| --- | --- | --- |
| D6 | Ultrasonic TRIG | Sends trigger pulse |
| A0 | Ultrasonic ECHO | Receives echo signal |
| A1 | IR Sensor OUT | Reads digital signal from IR sensor |
| D7 | Buzzer | Sound output |
| 5 V | All VCC pins | Power |
| GND | All GND pins | Ground reference |

## 💻 Arduino Code (from your file)

int buzzpin = 7;

int trigPin = 6;

int echoPin = A0;

int IRPin = A1;

int IRV;

int duration, distance;

void setup()

{

pinMode(trigPin, OUTPUT);

pinMode(buzzpin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(IRPin, INPUT);

Serial.begin(9600);

}

void loop()

{

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.034 / 2;

Serial.print("Distance in cm is: ");

Serial.println(distance);

if (distance < 20)

{

digitalWrite(buzzpin, HIGH);

}

else

{

digitalWrite(buzzpin, LOW);

}

delay(100);

IRV = digitalRead(IRPin);

Serial.print("IR sensor Value:");

Serial.println(IRV);

delay(1000);

}

## 🧠 Step-by-Step Code Explanation

### 1️⃣ Variable Declarations

int buzzpin = 7;

int trigPin = 6;

int echoPin = A0;

int IRPin = A1;

int IRV;

int duration, distance;

- buzzpin – controls the buzzer output.

- trigPin – sends ultrasonic trigger pulses.

- echoPin – receives echo signal from sensor.

- IRPin – reads digital signal from IR sensor.

- IRV – stores IR sensor reading (0 = object, 1 = no object).

- duration – time taken for echo to return.

- distance – calculated object distance in cm.

### 2️⃣ setup() – Initialization

pinMode(trigPin, OUTPUT);

pinMode(buzzpin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(IRPin, INPUT);

Serial.begin(9600);

- Sets each pin’s direction (input/output).

- Serial.begin(9600) starts communication with Serial Monitor at 9600 baud, allowing real-time readings.

### 3️⃣ Ultrasonic Distance Measurement

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

- Sends a 10 µs HIGH pulse on TRIG pin to emit ultrasonic waves.

- delayMicroseconds(2) ensures a clean LOW pulse before triggering.

- delayMicroseconds(10) ensures a valid trigger pulse width (≥ 10 µs).

### 4️⃣ Reading Echo and Calculating Distance

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.034 / 2;

- pulseIn() measures the time (in µs) that the ECHO pin stays HIGH—i.e., how long the wave takes to travel to the object and back.

- The formula:

Distance=Duration×0.0342\text{Distance} = \frac{Duration \times 0.034}{2}Distance=2Duration×0.034​

since sound speed ≈ 0.034 cm/µs and wave travels twice (there + back).

### 5️⃣ Display Distance

Serial.print("Distance in cm is: ");

Serial.println(distance);

Shows continuous distance readings in Serial Monitor.

### 6️⃣ Buzzer Control

if (distance < 20)

{

digitalWrite(buzzpin, HIGH);

}

else

{

digitalWrite(buzzpin, LOW);

}

delay(100);

- If the object is closer than 20 cm, buzzer turns ON (alert).

- Else, buzzer OFF.

- delay(100) (100 ms) prevents buzzing too rapidly and stabilizes readings.

👉 Why delay(100)?
To avoid multiple rapid toggles of the buzzer caused by small sensor fluctuations and to keep Serial Monitor updates human-readable.

### 7️⃣ IR Sensor Reading

IRV = digitalRead(IRPin);

Serial.print("IR sensor Value:");

Serial.println(IRV);

delay(1000);

- Reads IR sensor output:

  - 0 = Object detected

  - 1 = No object

- Prints status on Serial Monitor.

- delay(1000) (1 second) slows the loop for clearer observation.

👉 Why delay(1000)?
To provide enough time for the user (or the system) to see both distance and IR readings without flooding the Serial Monitor. Also helps in synchronizing sensor sampling rate.

## 🧩 Working Principle

- Ultrasonic Sensor

  - Emits ultrasonic sound from TRIG pin.

  - Measures time for echo to return to ECHO pin.

  - Arduino calculates distance.

- Buzzer

  - Turns ON if object distance < 20 cm → acts as alert mechanism.

- IR Sensor

  - Detects immediate proximity and prints digital value (0 or 1).

- Combined Function

  - Ultrasonic gives distance (quantitative).

  - IR gives presence (qualitative).

  - Buzzer gives alert (actuation).

  - This forms the base of an IIoT automation node—sensing, processing, and acting.

## 🖥️ Expected Serial Monitor Output

Distance in cm is: 35.27

IR sensor Value: 1

Distance in cm is: 18.56

IR sensor Value: 0

(When distance < 20 cm, buzzer sounds.)

## 🌐 Relation to IIoT

This setup demonstrates the “Sense → Analyze → Act” cycle used in industrial IoT:

- Sensors sense physical parameters (distance, presence).

- Arduino analyzes and decides based on thresholds.

- Actuator (buzzer) responds automatically.

- With networking modules (e.g., ESP8266 / NodeMCU), these readings can be sent to cloud dashboards for remote monitoring.

## ✅ Conclusion

Successfully implemented an IIoT-based robotics and automation prototype using:

- Ultrasonic sensor (distance measurement),

- IR sensor (obstacle detection),

- Buzzer (alert output).

Delays were used for sensor stabilization, signal processing, and readable output timing.
This circuit is foundational for autonomous robots, industrial safety systems, and smart monitoring applications.

## 🎤 Possible Viva Questions and Answers

| Question | Answer |
| --- | --- |
| What is IIoT? | Industrial Internet of Things – integration of IoT with industrial automation. |
| What is the function of the Ultrasonic Sensor? | Measures distance using reflected sound waves. |
| What is the purpose of the IR sensor? | Detects nearby object presence using reflected infrared light. |
| Why is a buzzer used? | To alert when an obstacle is too close. |
| What is the use of pulseIn()? | Measures the duration of a HIGH pulse on a pin (echo time). |
| Why divide by 2 in the distance formula? | Because sound travels to the object and back—round-trip distance. |
| Why delayMicroseconds(10)? | Required trigger pulse width for the Ultrasonic sensor. |
| Why delay(1000)? | To slow down loop execution for stable readings. |
| What happens if we remove delay(100)? | Buzzer and Serial Monitor will toggle too rapidly, causing unstable readings. |
| How can this be made IoT-enabled? | By adding Wi-Fi or MQTT module to transmit sensor data to the cloud. |