## 🎯 Aim

To design and implement an IIoT-based energy monitoring system using LDR (Light Dependent Resistor) and LED interfaced with Arduino to optimize energy consumption automatically based on ambient light levels.

## 🧩 Outcome

- Successfully interface LDR and LED with Arduino.

- Use sensor data (light intensity) to automate energy usage.

- Demonstrate the concept of smart energy management in IIoT systems.

## ⚙️ Hardware Requirements

| Component | Function |
| --- | --- |
| Arduino Uno | Main microcontroller board to read sensor data and control LED. |
| LDR (Light Dependent Resistor) | Senses ambient light intensity. |
| LED | Acts as a light source controlled by the system. |
| Resistor (10kΩ recommended) | Forms a voltage divider with LDR for analog voltage measurement. |
| Jumper Wires & Breadboard | Used for making connections. |

## 💻 Software Requirement

- Arduino IDE

## 📖 Theory

### 🔹 Industrial IoT (IIoT) Overview

IIoT (Industrial Internet of Things) combines sensors, microcontrollers, and data networks to monitor, optimize, and automate industrial systems.
In energy systems, IIoT helps by:

- Monitoring energy usage in real-time.

- Turning off unused lights/machines.

- Predicting maintenance needs and reducing waste.

### 🔹 Working Principle of LDR

An LDR (Light Dependent Resistor) changes its resistance based on the amount of light falling on it:

- More light → lower resistance → higher voltage output

- Less light → higher resistance → lower voltage output

This variable resistance is converted into a measurable analog voltage using a voltage divider circuit, which Arduino reads from an analog pin (A0).

The Arduino then compares this reading to a threshold to decide:

- Turn LED ON when it’s dark.

- Turn LED OFF when it’s bright.

## 🔌 Circuit Connections

| Component | Arduino Pin | Description |
| --- | --- | --- |
| LDR | A0 | Connected via voltage divider for analog input |
| LED (Anode) | D5 | Connected via resistor to Arduino output |
| LED (Cathode) | GND | Ground connection |
| Resistor (10kΩ) | Between A0 and GND | Creates voltage divider with LDR |
| LDR other leg | 5V | Power input |

## 💻 Arduino Code (from your file)

const int ledPin = 5; // digital pin 5

const int ldrPin = A0; // analog pin 0

void setup() {

Serial.begin(9600);

pinMode(ledPin, OUTPUT);

pinMode(ldrPin, INPUT);

}

void loop() {

int ldrStatus = analogRead(ldrPin);

if (ldrStatus <= 200){

digitalWrite(ledPin, HIGH);

Serial.print("Darkness over here, turn on the LED :");

Serial.println(ldrStatus);

}

else{

digitalWrite(ledPin, LOW);

Serial.print("There is sufficient light, turn off the LED : ");

Serial.println(ldrStatus);

}

}

## 🧠 Step-by-Step Code Explanation

### 1️⃣ Declaring Pins

const int ledPin = 5;   // LED is connected to digital pin 5

const int ldrPin = A0;  // LDR is connected to analog input pin A0

- const means the value will not change throughout the program.

- LDR gives an analog voltage (0–1023) that represents light intensity.

### 2️⃣ setup() Function

void setup() {

Serial.begin(9600);

pinMode(ledPin, OUTPUT);

pinMode(ldrPin, INPUT);

}

- Serial.begin(9600) starts serial communication at 9600 baud so we can monitor sensor readings in the Serial Monitor.

- pinMode(ledPin, OUTPUT) sets LED pin as output.

- pinMode(ldrPin, INPUT) sets LDR pin as analog input to read light intensity.

### 3️⃣ loop() Function

int ldrStatus = analogRead(ldrPin);

- Reads analog voltage (0–1023) from the LDR pin.

  - 0 = complete darkness

  - 1023 = maximum brightness

### 4️⃣ Decision Making

if (ldrStatus <= 200){

digitalWrite(ledPin, HIGH);

Serial.print("Darkness over here, turn on the LED :");

Serial.println(ldrStatus);

}

- If LDR reading ≤ 200 → it’s dark, so:

  - LED turns ON.

  - Prints light level (low value).

### 5️⃣ Otherwise

else {

digitalWrite(ledPin, LOW);

Serial.print("There is sufficient light, turn off the LED : ");

Serial.println(ldrStatus);

}

- If LDR reading > 200 → there’s enough light, so:

  - LED turns OFF to save energy.

  - Serial Monitor prints brightness status.

### ⚙️ No delay() in this code

Unlike other sensor codes, there’s no delay() used here, which means:

- The loop runs continuously, checking the LDR reading in real-time.

- This is ideal for instant response (LED turns ON/OFF immediately when light changes).

However, you can add a small delay for smoother Serial Monitor readings:

delay(500); // half-second delay for stable output

🔹 Reason for adding delay:
To reduce rapid flickering of the LED when the light level fluctuates around the threshold (200).

## 🖥️ Expected Serial Monitor Output

| Light Condition | LDR Reading | LED Status | Serial Output |
| --- | --- | --- | --- |
| Bright | 700–1023 | OFF | “There is sufficient light, turn off the LED : 890” |
| Dim/Shadow | 200–500 | ON/OFF (threshold) | “Darkness over here, turn on the LED : 180” |
| Dark | 0–200 | ON | “Darkness over here, turn on the LED : 90” |

## 🌐 IIoT Relevance

This simple setup demonstrates energy optimization, a key use-case of IIoT:

- Sensing: LDR collects environmental light data.

- Processing: Arduino analyzes it.

- Action: LED (or real light system) is controlled automatically.

- Scalability: Data can be sent to IoT dashboards (like Blynk, ThingsBoard, or ThingSpeak) for remote monitoring and analytics.

## ✅ Conclusion

Successfully implemented an IIoT-based energy monitoring system using Arduino, LDR, and LED.
The system detects ambient light and automatically controls lighting to conserve energy — a foundation for smart street lighting or industrial power optimization systems.

## 🎤 Viva / Oral Questions

| Question | Answer |
| --- | --- |
| What does LDR stand for? | Light Dependent Resistor. |
| What type of sensor is an LDR? | Analog sensor (resistance varies with light). |
| What is the function of the resistor in the LDR circuit? | Forms a voltage divider so Arduino can read analog voltage. |
| What is the analog value range in Arduino? | 0 to 1023 (10-bit ADC). |
| What is the threshold value used in the code? | 200 — below this LED turns ON. |
| Why is there no delay() in the program? | To allow real-time light detection; can be added for smoother readings. |
| What would happen if threshold = 800? | LED would remain OFF until it’s very dark. |
| How can this system be converted into a smart IoT system? | By adding Wi-Fi (ESP8266/ESP32) to upload light data to the cloud for monitoring. |
| What does Serial.begin(9600) do? | Starts serial communication at 9600 bits/second. |