To write a program for sending alert messages to the user by measuring temperature and humidity using the DHT11 sensor connected to Arduino.

## 🧩 Outcome

- Successful connectivity and configuration of the DHT11 temperature and humidity sensor with Arduino.

- Real-time measurement and display of environmental parameters (temperature in °C/°F and humidity in %).

- Understanding sensor-based IIoT communication using Serial Monitor.

## ⚙️ Hardware Requirements

| Component | Function |
| --- | --- |
| Arduino Uno | Microcontroller that processes sensor data. |
| DHT11 Sensor Module | Measures humidity and temperature digitally. |
| Jumper Wires | Used for circuit connections. |
| Breadboard | For prototyping the connections. |
| USB Cable | For power and communication between Arduino and PC. |

## 💻 Software Requirement

- Arduino IDE

## 📖 Theory

### 🔹 DHT11 Sensor Overview

- The DHT11 is a low-cost digital temperature and humidity sensor.

- It contains:

  - Thermistor for temperature measurement.

  - Capacitive humidity sensor for humidity measurement.

  - Signal conditioning circuit that converts analog signals into a digital data output.

### 🔹 Pin Configuration

| Pin | Name | Description |
| --- | --- | --- |
| 1️⃣ | VCC | Power Supply (3.3V to 5V) |
| 2️⃣ | DATA | Digital signal output (connects to Arduino pin) |
| 3️⃣ | NC | Not Connected |
| 4️⃣ | GND | Ground connection |

### 🔹 Working Principle

- The DHT11 sensor measures:

  - Humidity using a capacitive sensor.

  - Temperature using a thermistor.

- The microcontroller requests data from the sensor via a single-wire protocol.

- The sensor responds with digital data, which Arduino reads and converts into meaningful values (°C, °F, %RH).

- Data can be displayed on the Serial Monitor or sent to IoT dashboards (e.g., ThingSpeak, Blynk).

### 🔹 Applications

- Smart home temperature control systems.

- Weather monitoring stations.

- Greenhouse and agriculture monitoring.

- Industrial environmental monitoring.

## 💡 Circuit Connections

| DHT11 Pin | Arduino Pin | Description |
| --- | --- | --- |
| VCC | 5V | Power supply |
| GND | GND | Ground |
| DATA | D2 | Digital data signal |
| NC | — | Not used |

## 💻 Arduino Code (from your file)

#include "DHT.h"

#define DHTPIN 2       // Digital pin connected to the DHT sensor

#define DHTTYPE DHT11  // Sensor type DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

dht.begin();

}

void loop()

{

delay(2000);

float h = dht.readHumidity();

float t = dht.readTemperature();       // Celsius

float f = dht.readTemperature(true);   // Fahrenheit

Serial.print(F("Humidity: "));

Serial.print(h);

Serial.print(F("%  Temperature: "));

Serial.print(t);

Serial.print(F("°C "));

Serial.print(f);

}

## 🧠 Line-by-Line Code Explanation

### 1️⃣ Library and Constants

#include "DHT.h"

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

- #include "DHT.h" → Imports the library required to communicate with the DHT sensor.

- #define DHTPIN 2 → Defines Arduino digital pin 2 for DHT data communication.

- #define DHTTYPE DHT11 → Specifies sensor type (DHT11).

- DHT dht(DHTPIN, DHTTYPE); → Creates a sensor object named dht.

### 2️⃣ Setup Function

void setup() {

Serial.begin(9600);

dht.begin();

}

- Serial.begin(9600); → Starts communication between Arduino and Serial Monitor at 9600 baud rate.

- dht.begin(); → Initializes the DHT sensor to start data collection.

### 3️⃣ Loop Function

void loop()

{

delay(2000);

- delay(2000) = 2 seconds (important).
🔹 Reason for Delay:
The DHT11 sensor can only provide one reading every 2 seconds, as it needs time to stabilize between samples. Without this delay, readings might be inaccurate or return “NaN” (Not a Number).

### 4️⃣ Reading Humidity and Temperature

float h = dht.readHumidity();          // Reads humidity in %

float t = dht.readTemperature();       // Reads temperature in Celsius

float f = dht.readTemperature(true);   // Reads temperature in Fahrenheit

- Reads data from the sensor and stores values in variables h, t, and f.

- The true parameter converts temperature into Fahrenheit scale.

### 5️⃣ Printing Results

Serial.print(F("Humidity: "));

Serial.print(h);

Serial.print(F("%  Temperature: "));

Serial.print(t);

Serial.print(F("°C "));

Serial.print(f);

- Displays the humidity and temperature readings in Serial Monitor.

- The F() macro stores text in Flash memory, saving Arduino RAM.

## 🖥️ Expected Serial Monitor Output

Example output (every 2 seconds):

Humidity: 56.00%  Temperature: 28.00°C 82.40

Humidity: 57.00%  Temperature: 27.00°C 80.60

Humidity: 56.50%  Temperature: 28.50°C 83.30

## ⚙️ Explanation of Delay Importance

| Delay Function | Duration | Purpose |
| --- | --- | --- |
| delay(2000) | 2 seconds | Ensures the DHT11 sensor has enough time to read stable and accurate temperature/humidity data. Without this, data becomes noisy or invalid. |

## 🌐 How This Relates to IIoT

This is a basic IIoT environmental monitoring node:

- Sense: Temperature and humidity are sensed using DHT11.

- Process: Arduino processes and displays data locally.

- Act/Alert (optional): Threshold-based alerts can trigger fans, alarms, or send data to the cloud for remote monitoring.

For example:

If temperature > 35°C, send alert or activate cooling fan.

## ✅ Conclusion

Successfully interfaced DHT11 temperature and humidity sensor with Arduino.
Data is displayed through the Serial Monitor every 2 seconds.
This setup forms the foundation for IIoT-based environmental monitoring and alert systems.

## 🎤 Important Viva Questions

| Question | Answer |
| --- | --- |
| What is the full form of DHT? | Digital Humidity and Temperature Sensor. |
| What type of sensor is DHT11? | Digital sensor with a capacitive humidity and thermistor temperature sensor. |
| Why is delay(2000) used? | DHT11 updates its readings once every 2 seconds. Delay ensures accurate and stable data. |
| What are the units of output? | Humidity (%) and Temperature (°C and °F). |
| Which Arduino function reads the sensor data? | readHumidity() and readTemperature(). |
| What is the range of DHT11? | Temperature: 0–50°C, Humidity: 20–90% RH. |
| What happens if baud rate doesn’t match? | Serial Monitor displays gibberish/unreadable data. |
| How can this setup be extended for IIoT? | By connecting Arduino to Wi-Fi (e.g., ESP8266) and sending data to cloud platforms like ThingSpeak or Blynk. |